GreenTech Manufacturing – Evaluating Production Bottlenecks Through Downtime Analysis.

Introduction

GreenTech is a mid-size manufacturer that specializes in eco-friendly products. Challenges include optimizing production processes, particularly in maintaining efficient production schedules that minimize downtime and maximize throughput.

Throughput in manufacturing is the amount of product a company produces within a specific timeframe, measured from raw materials all the way through to finished products, that is, if a factory fabricates 100 chairs over 10 days, its throughput is 10 chairs per day. Formula = total output / total time.

Factors that contribute to downtime:

- Machine Failures

- Material Shortages

- Inefficient Production Planning

- Manual Scheduling: current system is highly reliant on manual input

- Limited data visibility

Impact of downtime:

- Operational Inefficiency

- Financial Costs

- Customer Delays

- Resource wastage

The main objective is to optimize production schedules to minimize downtime, reduce operational costs, and increase overall efficiency.

Specific objectives:

- Identify root causes for downtime

- Optimize production scheduling

- Improve Operator Allocation

- Enhance Reporting Transparency

- Enable Continuous Improvement

Percentage of downtime = (total downtime/planned production time) * 100.

Mean time to repair (MTTR): how quickly the team repairs breakdowns

Mean time between Failures (MTBF): how long a piece of equipment runs between failures

OEE shows how efficiently a piece of equipment is operating compared to its full potential.

To calculate the direct financial cost of downtime:

(lost production * unit value) + labour cost + overhead cost.

Methodology

After injecting the data, the tables in the database were all assessed to get a better understanding of the data structure. The data contained 4 tables: ‘downtime_factors’, ‘line_downtime’, ‘line_productivity_batches’, and ‘products.’ The downtime_factors table has 13 different factors that could cause a downtime, these factors also included operator error. The line_downtime table consists of information about the number of minutes of each factor that contributes to the downtime in a particular batch and where a particular factor does not contribute to a downtime in that batch, it is recorded as null. There are 282 batches in the dataset where all factors resulting in downtime are null values indicating that these batches had no downtime. The line_productivity_batches table links to the other tables with the foreign keys: Product_ID and Batch_ID, and it also gives insight into how long a batch of production takes. The Date column and date in the Start_Time column are the same when compared therefore one of them will be dropped. The product table contains information about the product the company produces, there are 4 of them: Ecowash Liquid Detergent, Biowipe Cleaning Sheets, Greenfoam Hand Soap, and Repack Recycled Packaging Films.